# Channel Model v2 — Testing and Validation Report

Scope: numerical/physics-consistency and implementation validation for the directional UAV mmWave channel model used in the ABPO experiment.

# Configuration

Carrier=28.0 GHz; Bandwidth=100.0 MHz; NF=7.0 dB; beamwidth=5.0–60.0 deg; power budget=1.000 W; sidelobe floor=-20.0 dB.

# Validation results

| Test | Status | Detail |
| --- | --- | --- |
| FSPL monotonic with distance | PASS | [87.40553272617268, 93.4261326394523, 99.44673255273193, 105.46733246601156] |
| Channel gain decreases with distance | PASS | [1.8173841134826242e-09, 4.5434602837065596e-10, 1.1358650709266396e-10, 2.839662677316598e-11] |
| Boresight gain decreases with beamwidth | PASS | [72.0, 36.0, 18.0, 9.0, 6.0] |
| Pointing mismatch reduces gain | PASS | [36.0, 32.25869352787294, 18.18, 2.5875, 0.36054382324218753] |
| Sidelobe floor enforced | PASS | [36.0, 32.25869352787294, 18.18, 2.5875, 0.36054382324218753] |
| Beam-training fraction bounded | PASS | [0.982, 0.991, 0.9955, 0.99775, 0.9985] |
| Rate equation finite/non-negative | PASS | [402489918.5734418, 415771810.9354002, 409378781.41549116, 421971191.13126636] |
| NSE equation consistency | PASS | (16.496117020555996, 16.496117020555996) |
| SINR finite | PASS | [15.48512709562544, 17.082376776207138, 16.295120175802293, 17.879994644329166] |
| Per-step total power budget | PASS | 1.0 |
| Per-link minimum power | PASS | [0.25, 0.25, 0.25, 0.25] |

# Interpretation

The tests verify internal numerical consistency: distance/path-loss behavior, directional gain, pointing mismatch, training overhead, finite SINR/rate, NSE consistency, and power feasibility. These tests do not constitute empirical validation against measured 28-GHz propagation.